(2061) MATERIAL DELIVERY MANAGEMENT SYSTEM (Bituminous and Concrete)

REVISED 06/24/26 ◄DO NOT REMOVE THIS.  IT NEEDS TO STAY IN FOR THE CONTRACTORS.

Use when Pay item is on the Project.

- Do not use on IDIQ Projects.

Always use the following write-ups with this write-up:

- SP2301 Concrete Paving (MDMS) and/or

- SP2461 Structural Concrete (MDMS)

SP2020-78.1

DESCRIPTION

This work will consist of collecting and managing Source, Hauler, and Loading and Delivery Event Data related to material deliveries for the Contract. All data must be captured in a standardized electronic format.

Definitions

For the purpose of the Work specified in section S-113, “Material Delivery Management System,” the Department defines:

AASHTOWare Project (AWP)

Web-based software suite that supports all phases of the construction lifecycle – from project definition through cost estimation, bidding/letting process, and construction and materials management. It uses a unified database to support reporting, decision-making, and historical tracking.

Agency MDMS Verification

A daily process where the Engineer randomly verifies MDMS data to ensure system accuracy and to confirm that final exported data matches what was reviewed in the field.

Application Programing Interface (API)

Software interface that allows different systems to communicate and exchange data.

Breadcrumb Trail

A series of GPS coordinates with time stamps that show and record a truck’s location at set time intervals.

Category

Project subsection associated with a specific funding source or contributor.

Contract Geofence

Fixed virtual boundary surrounding the limits of Contract/jobsite work.

Contract Total by Material Code Quantity

Cumulative quantity for each Material Code under the Contract.

Control Section

Defined segment of the state highway system used for recordkeeping, construction, maintenance, and administrative purposes. These segments use fixed termini, usually at intersections with other trunk highways, county lines, or other geographical features.

Daily Running Total by Material Code Quantity

Cumulative daily quantity for each Material Code under the Contract.

Dump

The act of delivery and placing Source material inside the Contract Geofence.

E-Ticket (Source Data)

Electronic version of Source Data, provided in a standardized, transferable format such as JSON via APIs.

Geofence (Geographic Boundary)

Virtual geographic boundary that identifies when a device enters or exits a defined area.

Hauler Data

Data provided by the hauler, including truck identification, driver name, broker name, DOT number, and other required data. The hauler for the given truck identification may be the Contractor, independent truck operator (ITO), or managed truck operator (MTO).

Hypertext Transfer Protocol (HTTP)

Communication protocol used to transmit data over the internet.

Intelligent Construction Technology (ICT) Contract Portfolio

A VetaCenter container that stores and manages ICT data from one or more ICT Platform Workspace for a specific Contractor and Contract.

Intelligent Construction Technology (ICT) Federated Contract

A higher-level VetaCenter container that stores and manages ICT data from multiple ICT Contract Portfolios.

Intelligent Construction Technology (ICT) Platform

Software or applications that collect, store and analyze data from technologies such as the Material Delivery Management System (MDMS), Intelligent Compaction (IC), Paver Mounted Thermal Profiling (PMTP), Dielectric Profile System, and others.

Intelligent Construction Technology (ICT) Platform Workspace

A VetaCenter container that stores and manages ICT data pushed from a specific ICT platform (Custodian User Account) for a specific Contractor and Contract.

Item Identification

Unique identifier assigned to each pay item in the specification reference list.

JavaScript Notation (JSON)

Lightweight, human-readable data-interchange format used for MDMS data transfer.

Loading and Delivery Event Data

Data generated during material pickup and delivery, such as Dump details, date and time stamps, and durations.

Material Code

Unique identifier for each construction material; sometimes also shown with the material description.

Material Delivery Management System (MDMS)

Electronic platform that manages Source, Hauler, and Loading and Delivery Event Data associated with delivery of material for a Contract.

Mix Design Identification

Unique identifier for a mix design such as a mix designation report number or sheet number.

OAuth 2.0

Industry standard protocol used to authorize secure API access.

Overweight Weight

Weight of the load that exceeds the maximum allowable gross weight of the transport vehicle.

Paper Weight Ticket

Also called Bill of Lading. Printed copy of the weight ticket from the loadout system; includes Certificate of Compliance for S-XX (2461) Structural Concrete (Material Delivery management System) and computerized batch tickets for S-XX (2301) Concrete Pavement (Material Delivery management System).

Representational State Transfer (REST)

Architectural style that applies standards in the HTTP protocol creating the capability of exposing APIs over the internet.

RESTful

Term for software built in the REST style which requires the following 6 guiding constraints: 1) client server; 2) stateless; 3) cacheable; 4) uniform interface; 5) layered system; 6) code on demand (optional).

Source

Original material provider for the Contract.

Source Data

Data generated by the Source’s loadout system, including Contract, project, Source, and Mix Design Identification, Material Code, ticket identification, and loading and weight information. This data is considered the E-Ticket.

Source Geofence

Virtual geographic boundary around the Source – such as a boundary around plant.

Source Identification

The Source’s plant/pit name or identifier as recognized by AASHTOWare Project.

Split Load

Load whose material is delivered to more than one location, such as for patching, entrances, multiple piers, multiple intersections for curb and gutter, and other locations.

Supplier

Material producer serving multiple Contractors.

System Failure

Any instance where the MDMS is unable to collect, store, and/or push required data to VetaCenter.

Test E-Ticket

Simulated E-Ticket containing required Contract-specific data, used to verify MDMS data mapping and data transfer into VetaCenter.

Truck Driver Classification

Description of the driver classification as defined by the Federal Wage System. For example: Tractor Trailer Driver; Four or More Axle Unit, Straight Body Truck; Three Axle Units, or Two Axle Unit.

VetaCenter

The Departments centeralized platform used to store, map and analyze intelligent construction technology (ICT) data, including MDMS data.

MATERIALS - BLANK

CONSTRUCTION REQUIREMENTS

Use the MDMS for Work performed under the following specifications:

Update provision to include 2461 and/or 2301.

- 2360, “Plant Mixed Asphalt Pavement”

- 2363, “PASSRC and PASB”

- 2365, “Stone Matrix Asphalt”

- 2461 “Structural Concrete” when greater than or equal to 500 cubic yards

- 2301 “Concrete Pavement”

Collection of the following MDMS data is not required when the total estimated contract quantity of each bituminous mix type is 600 tons or less:

- Truck location tracking per S-113.3.A(5)

- Loading and Delivery Event Data per Table SP2061-12

- Dump event (date, time, and location) per S-113.3.A(7), and S-113.3.B.4.

- Source and Contract Geofences per S-113.3.A(6) and S-113.3.B.3.

Equipment and Software – Contractor’s MDMS

Use a MDMS that meets the following requirements:

Source Data Integration

The system will digitalize Source Data and standardize data fields generated from varying loadout software platforms to create the E-Ticket. See Table SP2061-1 for required Source Data fields and standardized naming conventions.

Source Identification and Truck Editing

The system will allow users to enter Source Identification data, per Table SP2061-9, and edit truck identification in the E-Ticket if trucks order changes during loading process.

Hauler Data Management

The system will allow users to enter or import all Hauler Data, per Table SP2061-11, for each E-Ticket.

Truck Location Tracking Equipment

Provide portable, or hardwired, equipment capable of tracking locations of trucks delivering materials for 2118, “Aggregate Surfacing”; 2360, “Plant Mixed Asphalt Pavement”; 2363, “PASSRC and PASB”; and 2365, “Stone Matrix Asphalt”. The tracking system will:

(a)	Operate independently or via adapter power.

(b)	Indicate signal interruptions preventing the collection of truck locations.

(c)	Associate data with corresponding truck and ticket identification.

(d)	Record Breadcrumb Trail data at intervals of 1 minutes or less.

(e)	Allow playback of truck transit routes for each Breadcrumb Trail.

Static Geofence

Establish Source and Contract Geofences for trucks delivery materials for 2118, “Aggregate Surfacing”; 2360, “Plant Mixed Asphalt Pavement”; 2363, “PASSRC and PASB”; and 2365, “Stone Matrix Asphalt”. The system will record Geofence entry and exit data per the requirements of Table SP2061-12.

Dump Event Triggering

The system will include an automated method to trigger and record Dump times and locations per the requirements of Table SP2061-12.

Source Data Requirements

(a)	Suppliers and Contractor owned Sources will provide Source Data to the MDMS in 2 minutes or less from point of sale.

(b)	The MDMS will display Source Data (data fields contained in Table SP2061-1) to users within one minute of receipt of data.

(c)	Contractor-owned permanent and portable Sources will provide Source Data through a solution of the MDMS or per Section S-113.3.A.7(d).

(d)	Suppliers and Contractor-owned permanent and portable Sources will provide Source Data to the MDMS using:

(1)	REST APIs with OAuth 2.0 Standard

(2)	JSON formatting for data transfer

(3)	JSON request body format will contain the Source Data per ticket per Table SP2061-1.

(4)	The MDMS will support multiple ticket transfers in a single JSON message when batch queuing occurs.

(e)	The Contractor’s MDMS will push Source, Hauler, and Loading and Delivery Event data to VetaCenter, using REST APIs and a JSON data interchange language, in 4 minutes or less from point of sale.

Loading and Delivery Event Data Transfer

The system will push all Loading and Delivery Event data to VetaCenter within 1 minute after each event using REST APIs and a JSON.

Export Capabilities

System will allow the Engineer to export Source, Hauler, and Loading and Delivery Event Data (per Tables SP2061-1, SP2061-10, and SP2061-12) in dbase ASCII, CSV, XLSX, or text format. Exports will be available at intervals not exceeding 15 minutes.

Loading and Delivery Event Data collected in systems separate from the MDMS must include ticket identification, Source Identification, and loading date and time to allow cross referencing to Source and Hauler Data.

Engineer Access

Engineer has ability to view MDMS data in Contractor’s/Suppliers platform in 3 minutes or less from the point of sale (using a web- or application-based user interface) when adequate data cellular coverage is available. This includes the following information:

(a)	Truck counts by status (at Source, in transit from Source to Contract, at Contract (and/or Dump), and in transit from Contract to Source)

(b)	Ticket status summary (ticket identification, loaded, in transit, Dumped)

(c)	Source Data per Tables SP2061-1 and SP2061-9

(d)	Hauler Data per Table SP2061-10

(e)	Loading and Delivery Event Data per Table SP2061-12

(f)	Access to playback of truck transit routes for each Breadcrumb Trail

Instrumentation

Instrument-appropriate components of the MDMS to collect data per Tables SP2061-1, SP2061-10, and SP2061-12.

Table SP2061-1

Required Source (E-Ticket) Data Fields in Contractor’s MDMS for each Data Block

| JSON Field Name | Long Description | Examples | Data Type | Unit Category | Data Field Required for: MnDOT 2118 | Data Field Required for: MnDOT 2360, MnDOT 2363, MnDOT 2365 | Data Field Required for: MnDOT 2301 and MnDOT 2461 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ticketId | Ticket Identification | 5126349, 101R, 539-19 | String (up to 20 characters) | None | Yes | Yes | Yes |
| contractId | Contract Identification | 180181, R-37463 | String (up to 20 characters) | None | Yes | Yes | Yes |
| projectId | Agency State Project (SP) / State Aid Project (SAP) Identification | 1234-56, 47-609-012 | String (up to 20 characters) | None | Yes | Yes | Yes |
| contractorName | Contractor Name | Al Fresco Contracting | String (up to 256 characters) | None | Yes | Yes | Yes |
| sourceId | Source Identification | BP001, RM001, GS79103 (e.g., pit / plant identification) | String (up to 20 characters) | None | Yes | Yes | Yes |
| scaleId * | Scale Identification | 2, A2 | String (up to 20 characters) | None | Yes | Yes | No |
| siloId * | Silo Identification | 5, A3 | String (up to 20 characters) | None | No | Yes | No |
| sourceOperatorName | Source Operator Name | Adam Zapel (e.g., weighmaster) | String (up to 256 characters) | None | Yes | Optional | Optional |
| sourceNote | Source Notes | First Load, Last Load, Warnings | String (up to 3999 characters) | None | Optional | Optional | Optional |
| mixDesignId | Mix Design Identification | 02-2020-184 (i.e., mix design report number) RMS135-030, JMF25-001, PS25-002 (e.g., mix design sheet number) | String (up to 40 characters) | None | No | Yes | Yes |
| materialCode | Material Code | SPWEA340C, 3F52, Class 5, (e.g., material description, mix number, classification) | String (up to 20 characters) | None | Yes | Yes | Yes |
| loadNumber ║ | Load Number | 75 | Number | None | Yes | Yes | Yes |
| truckId † | Truck Identification | 51.6046, 88tb, T-1, T1 | String (up to 20 characters) | None | Yes | Yes | Yes |
| trailerId † | Trailer Identification | 51.6046, 88tb, T-1, T1 | String (up to 20 characters) | None | Yes | Yes | Yes |
| voidedTicket | Voided Ticket | See Table SP2061-2 | String (up to 10 characters) | None | Yes | Yes | Yes |
| loadDateTime ‡ # | Loading Date and Time | 2007-04-05T12:30:45-02:00 | String (up to 40 characters) | Time | Yes | Yes | Yes |
| grossWt § ** | Gross Weight | 44.33 | Number | Mass | Yes | Yes | No |
| netWt § | Net Weight | 26.83 | Number | Mass | Yes | Yes | No |
| truckTareWt § ** | Truck Tare Weight | 17.5 | Number | Mass | Yes | Yes | No |
| dailyRunningTotalByMatlCodeQty § †† | Daily Running Total By Material Code Quantity | 1900.64, 11 | Number | Mass or Volume | Yes | Yes | Yes |
| contractTotalByMatlCodeQty § †† | Contract Total by Material Code Quantity | 2400.45, 22 | Number | Mass or Volume | Optional | Optional | Optional |
| batchedQty § | Batched Quantity | 11 | Number | Mass or Volume | No | No | Yes |
| ingredientCode ║║ | Ingredient Code | See Table SP2061-4 | String (up to 30 characters) | None | No | No | Yes |
| aggregateSize | Aggregate Size | ¾” +, ¾” - , #67, Sand, CIA (e.g., aggregate size/fraction) | String (up to 15 characters) | None | No | No | Yes |
| ingredientSourceName ║║ | Ingredient Source Name | See Table SP2061-8 for Ingredient Source Names for Water. STGBMOIL, COCUNND, AMPOL1020, MAIR90, 19128, 19128, 73006 | String (up to 50 characters) | None | No | No | Yes |
| targetBatchIngredientQty § ║║ †† | Target Batch Ingredient Quantity | 2685, 9, 24.5 | Number | Varies | No | No | Yes |
| actualBatchedIngredientQty § ║║ †† | Actual Batched Ingredient Quantity | 2685, 9, 24.6 | Number | Varies | No | No | Yes |
| batchedQtyVarience § ║║ | Batched Quantity Variance | -2.11, -0.37, 0.5, 1.34 | Number | Percent | No | No | Yes |
| aggregateAbsorptionPercent | Aggregate Absorption Percent | 1.2 | Number | Percent | No | No | Yes |
| totalAggregateMoisturePercent | Total Aggregate Moisture Percent | 3.3 | Number | Percent | No | No | Yes |
| waterAddedSourceQty § | Water Added at Source Quantity | 3 | Number | Mass or Volume | No | No | Yes |
| waterAvailableAddQty § | Water Available to Add Quantity | 34.992 | Number | Volume | No | No | Yes |
| waterToCementitiousRatioAfterWaterAddedAtSource | Water to Cementitious Ratio after Water Added at Source | 0.38 | Number | None | No | No | Yes |
| maximumDesignWaterToCementitiousRatio | Maximum Design Water to Cementitious Ratio | 0.45 | Number | None | No | No | Yes |
| qualityControlId ‡‡ | Quality Control Identification | Dan D. Lyon | String (up to 256 characters) | None | No | No | Yes |
| qualityControlTechnicalCertificationId ‡‡ | Quality Control Technical Certification Identification | 12345SX | String (up to 20 characters) | None | No | No | Yes |
| * Scale and silo identifications are required only when multiple scales and silos are used at the Source. ║ Load numbers must be sequential and may not be shared sequences between projects. † Truck and Trailer identifications must match the physical identification labels visible on the equipment. Trailer is considered a “pup trailer”. Trailer ID is not required for tractor-trailer combinations. ‡ Date and time values must include the UTC offset, using the format: YYYY-MM-DDThh:mm:ss±hh:mm. # For concrete, the loading date/time represents the batch time when water is added to the cementitious materials. § See Table SP2061-3 for lookup table for units of measurement. Include unit of measurement in JSON data interchange language (see S-113.3.A(7)(d)). ** Required when a load cell is not used on hoppers beneath a surge or storage bin. ║║ Up to 14 ingredient codes and associated ingredient Source names, target batch ingredient quantities, actual batched ingredient quantities, and batch quantity percent variances. Push fields as an array in JSON coding. †† Provide saturated surface dry (SSD) condition quantity for concrete aggregates. Concrete aggregate is measured by mass. The quantity of aggregate weighed is the required dry mass plus the total moisture content (absorbed and surface) of the aggregate. ‡‡ Provide data when data field is available in MDMS. |  |  |  |  |  |  |  |

Table SP2061-2

Lookup Table for Voided Tickets *

Description

Voided – Source ticket identification voided.

Valid – Material loaded for delivery to project.

* Data block value is provided by database as an identifier.

Table SP2061-3

Lookup Table for Units of Measurement *

| Unit Category | Description |
| --- | --- |
| Volume | Cubic Meter |
| Volume | Cubic Yard |
| Volume | Gallons |
| Mass | Kilogram |
| Mass per Volume | Kilograms per Cubic Meter |
| Mass | Metric Tons |
| Length | Millimeters |
| Length per Mass | Millimeter per Kilogram |
| Volume | Ounces |
| Volume per Volume | Ounces per Cubic yard |
| Volume per Mass | Ounces per 100 Pound Cementitious |
| Mass | Pounds |
| Mass per Volume | Pounds per Cubic Yard |
| Area | Square Yard |
| Area per Length | Square Yard per Inch |
| Mass | US Tons |
| * Data block value is provided by database as an identifier. |  |

Table SP2061-4

Lookup Table for Ingredient Code

| Description | Data Block Value |
| --- | --- |
| Additive / Admixture | See Table SP2061-5 |
| Aggregate | See Table SP2061-6 |
| Cementitious | See Table SP2061-7 |
| Water | Water * |
| * Data block value is provided by database as an identifier. |  |

Table SP2061-5

Lookup Table for Ingredient Code for Additives and Admixtures *

Description

Air-Entraining Admixtures ║

Calcium Chloride

Color

Fibers

Glass

Ground Tire Rubber

Lime

Liquid Anti-Stripping

Plastic

Recycling Agent

Recycled Asphalt Shingle

Type A—Water-Reducing Admixtures †

Type B—Retarding Admixtures †

Type C—Accelerating Admixtures †

Type D—Water-Reducing and Retarding Admixtures †

Type E—Water-Reducing and Accelerating Admixtures †

Type F—Water-Reducing, High-Range Admixtures †

Type G—Water-Reducing, High-Range, and Retarding Admixtures †

Type S—Specific Performance Admixtures †

Warm Mix Additive

* Data block value is provided by database as an identifier. ║ AASHTO M154 Standard Specification for Air-Entraining Admixtures for Concrete † ASTM C 494 Standard Specification for Chemical Admixtures for Concrete

Table SP2061-6

Lookup Table for Ingredient Code for Aggregate *

Description

Coarse Aggregate

Fine Aggregate

Intermediate Aggregate

Recycled Asphalt Pavement

Recycled Concrete Aggregate

* Data block value is provided by database as an identifier.

Table SP2061-7

Lookup Table for Ingredient Code for Cementitious *

Description

Asphalt Cement

Blended Hydraulic Cement

Fly Ash

Metakaolin

Natural Pozzolan

Portland Cement

Silica Fume

Slag

* Data block value is provided by database as an identifier.

Table SP2061-8

Lookup Table for Ingredient Source Names for Water *

Description

Clarified Water

Cold Water

Hot Water

Reclaimed Water

Temper Water

Wash Out Water

* Data block value is provided by database as an identifier.

Table SP2061-9

Required Fields in Contractor’s MDMS for Source Identification Data

| JSON Field Name | Long Description | Examples | Data Type |
| --- | --- | --- | --- |
| sourceId | Source Identification | BP001, RMX001, GS79103 (e.g., pit, plant identification) | String (up to 20 characters) |
| sourceName | Source Name | John Asphalt, John Asphalt 3 Burnsville, John Plant 7 (e.g., long description of plant name) | String (up to 256 characters) |
| sourceAddress | Source Address | 12345 Marvel Street NW, MN 56738 | String (up to 3999 characters) |
| sourcePhoneNumber | Source Phone Number | 777-777-7777 | String (up to 20 characters) |

Table SP2061-10

Required Hauler Data Fields in Contractor’s MDMS for each Data Block

| JSON Field Name | Long Description | Examples | Data Type | Unit Category | Data Field Required for: MnDOT 2118, MnDOT 2360, MnDOT 2363, MnDOT 2365, and MnDOT 2301 | Data Field Required for: MnDOT 2461 |
| --- | --- | --- | --- | --- | --- | --- |
| ticketId | Ticket Identification | 5126349, 101R, 539-19 | String (up to 20 characters) | None | Yes | No |
| sourceId | Source Identification | BP001, RMX001, GS79103 (e.g., pit, plant identification) | String (up to 20 characters) | None | Yes | No |
| loadDateTime * | Loading Date and Time | 2007-04-05T12:30:45-02:00 | String (up to 40 characters) | Time | Yes | No |
| haulerCompanyName | Hauler Company Name | Don Key Contracting | String (up to 256 characters) | None | Yes | No |
| brokerName ║ | Broker Name | Max E Mumm | String (up to 256 characters) | None | Optional | No |
| truckDotNumber # | Truck DOT Number | US DOT 33136 | String (up to 20 characters) | None | Yes | No |
| truckId † | Truck Identification | 51.6046, 88tb, T-1, T1 | String (up to 256 characters) | None | Yes | No |
| truckDriverClass ‡ # | Truck Driver Classification | See Table SP2061-11 | String (up to 50 characters) | None | Optional | No |
| overweightPermitNumber # | Overweight Permit Number | 033I9021331 | String (up to 20 characters) | None | Optional | No |
| maximumGrossWt | Maximum Gross Weight | 44 | Number | Mass | Yes | No |
| driverName ║ | Driver Name | Stan Dupp | String (up to 256 characters) | None | Optional | No |
| * Date and time values must include UTC offset and follow the format: YYYY-MM-DDThh:mm:ss±hh:mm. ║ Data field required only for MTO / ITO haulers. † Truck Identification must match the Truck Identification used on the E-Ticket (Source Data). ‡ Tie Truck Driver Classification to truck identification. # Provide data when data field is available in MDMS. |  |  |  |  |  |  |

Table SP2061-11

Lookup Table for Truck Driver Classification *

Format

Tractor Trailer Driver

Four or More Axle Unit, Straight Body Truck

Three Axle Units

Two Axle Unit

* Data block value is provided by database as an identifier.

Table SP2061-12

Required Loading and Delivery Event Data Fields in Contractor’s MDMS for each Data Block

| JSON Field Name | Long Description | Examples | Data Type | Unit Category | Data Field Required for: MnDOT 2218 | Data Field Required for: MnDOT 2360, MnDOT 2363, MnDOT 2365 | Data Field Required for: MnDOT 2301 | Data Field Required for: MnDOT 2461 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ticketId | Ticket Identification | 5126349, 101R, 539-19 | String (up to 20 characters) | None | Yes | Yes | Yes | Yes |
| sourceId | Source Identification | BP001, RMX001, GS79103 (e.g., pit/plant identification) | String (up to 20 characters) | None | Yes | Yes | Yes | Yes |
| loadDateTime * | Loading Date and Time | 2007-04-05T12:30:45-02:00 | String (up to 40 characters) | Time | Yes | Yes | Yes | Yes |
| geofenceType | Geofence Type | See Table SP2061-13 | String (up to 10 characters) | None | Yes | Yes | No | No |
| startDateTime * ║ | Start Date and Time | 2007-04-05T12:30:54-02:00 | String (up to 40 characters) | Time | Yes | Yes | No | No |
| endDateTime * ║ | End Date and Time | 2007-04-05T12:30:54-02:00 | String (up to 40 characters) | Time | Yes | Yes | No | No |
| startLatitude ║ | Start Latitude | 47.863023 | Number | None | Yes | Yes | No | No |
| startLongitude ║ | Start Longitude | -92.870154 | Number | None | Yes | Yes | No | No |
| endLatitude ║ | End Latitude | 47.865479 | Number | None | Yes | Yes | No | No |
| endLongitude ║ | End Longitude | -93.081257 | Number | None | Yes | Yes | No | No |
| dumpDateTime * † | Dump Date and Time | 2007-04-05T12:30:54-02:00 | String (up to 40 characters) | Time | No ‡ | Yes # § ** | No ‡ | No ‡ |
| dumpLatitude | Dump Latitude | 47.858555 | Number | None | No ‡ | Yes # § ** | No ‡ | No ‡ |
| dumpLongitude | Dump Longitude | -92.962875 | Number | None | No ‡ | Yes # § ** | No ‡ | No ‡ |
| dumpMachineId | Dump Machine Identification | SX43201J, Mainline Paver | String (up to 256 characters) | None | No ‡ | Yes # § ** | No ‡ | No ‡ |
| spatialReferenceAuthority | Spatial Reference Authority | EPSG | String (up to 20 characters) | None | No ‡ | Yes | No ‡ | No ‡ |
| spatialReferenceId | Spatial Reference Identification | 4326 | Number | None | No ‡ | Yes | No ‡ | No ‡ |
| startDumpTime * ║║ §§ | Time at Start of Dump (Discharge) | 2007-04-05T12:30:54-02:00 | String (up to 40 characters) | Time | No | No | No | Yes |
| endDumpTime * ║║ §§ | Time at End of Dump (Final Discharge) | 2007-04-05T12:30:54-02:00 | String (up to 40 characters) | Time | No | No | No | Yes |
| durationInTruck ║║ †† ‡‡ ## | Duration of time in Truck | 00:15:30 | String (up to 40 characters) | Time | No | No | No | Yes |
| * Date and time values must include UTC offset using the format: YYYY-MM-DDThh:mm:ss±hh:mm. ║ Start and end times and latitude and longitude for Geofence events are recorded upon entering and exiting the Contract and Source Geofences. † The dump date/time value is determined by the drum rotation sensors (End of Dump) for ready-mixed concrete applications. ‡ Recorded using VetaCenter as needed. # Dump Date and Time, Dump Latitude and Dump Longitude, and Dump Machine Identification must be recorded using an automated triggering mechanism. § Recording of Dump event data is not required for placement of bituminous mixtures used for the following: - Patching - Filling areas of excavation that are less than 750 linear feet - Tight blade leveling - Bituminous trails ** Record Dump event data for echelon paving applications as follows: - Bituminous mixture produced from two different Sources: use the automated triggering mechanism associated with the paving train, to which the bituminous mixture is being delivered, to record Dump location for the delivered mixture. - Bituminous mixture produced from single Source: use the automated triggering mechanism associated with either paving train to record Dump event data for the delivered mixture. ║║ Ready-mixed concrete Supplier will send startDumpTime, endDumpTime, and durationInTruck to the Contractor's MDMS. †† "Duration of time in truck" equals the duration of time between the Time at End of Dump and the Loading Date and Time (durationInTruck = endDumpTime - loadDateTime) ‡‡ Use the following format: HH:MM:SS. ## Provide data when data field is available in MDMS. |  |  |  |  |  |  |  |  |

Table SP2061-13

Lookup Table for Geofence Type *

Description

Contract – Limits of jobsite.

Source – Location where material is loaded.

* Data block value is provided by database as an identifier.

Preconstruction Activities

Complete the following prior to delivery material:

Source Identification and Hauler Data

Enter Source Identification data (per Table SP2061-9) into the MDMS along with other needed startup information.

Enter Hauler Data (per Table SP2061-10) into VetaCenter.

Internet (or Satellite) Connectivity

Set up internet (or satellite) connectivity at each Source.

Geofences

Create the required Source and Contract Geofences in the MDMS when delivery materials for 2118, “Aggregate Surfacing”; 2360, “Plant Mixed Asphalt Pavement”; 2363, “PASSRC and PASB”; and 2365, “Stone Matrix Asphalt”. Create the Source Geofence(s) so that it includes the roadway or area outside the Source where trucks queue before entering for loading.

Provide Engineer with snapshot of map showing the Source and Contract Geofence boundaries.

Dump Event

Set up trigger for recording Dump time and location per requirements of Table SP2061-12.

ICT Workspaces and Test E-Tickets

Engineer will create ICT Contract Portfolio in VetaCenter.

Create and set up the ICT Platform Workspace in VetaCenter to connect with ICT Platform. Map MDMS data fields to VetaCenter for each Source providing material to Contract.

Send a Test E-Ticket for each Source to VetaCenter no later than 21 Calendar Days prior to start of Work requiring MDMS. The Test E-Ticket must:

(a)	Use valid project identifies

(b)	Set Voided Ticket to “Voided”

(c)	Set Net Weight as 1 ton for bituminous or Batched Quantity as 1 cy for concrete.

Notify Engineer when Test E-Ticket is sent to VetaCenter for review and acceptance.

Delivery of material from Source before acceptance of Test E-Ticket(s) by the Engineer is a Hold Point and constitutes Unauthorized Work in accordance with 1503, “Conformity with Contract Documents” and 1512, “Unacceptable and Unauthorized Work.”

Training

Provide training to Engineer no later than 7 calendar days prior to start of work requiring MDMS.

Training will cover:

(1)	MDMS web- or application-based platforms.

(2)	Data fields included in Contractor’s MDMS data collection and export.

(3)	Breadcrumb Trails playback instructions.

(4)	Example data exports per section S-113.3.A(9).

MDMS Data

Provide Engineer access to the MDMS prior to start of delivery of material and until 90 days after final acceptance of all work.

Collect and push MDMS data to VetaCenter per requirements of this provision.

System Failure

Notify Engineer upon discovery of System Failure and after resolution of System Failure.

Source will revert to other means and methods accepted by the Engineer, for sharing Source data during System Failures. Contractor is allowed to deliver material from Source using the means and methods approved by the Engineer until the start of the next Working Day. The Engineer will not allow the Contractor to deliver material from the Source on the next Working Day until the System Failure is remedied to the satisfaction of the Engineer.

Documentation

Provide Engineer Paper Weight Ticket(s) per requirements for 2118, “Aggregate Surfacing”; S-XX (2301) Concrete Pavement (Material Delivery management System); 2360, “Plant Mixed Asphalt Pavement”; 2363, “PASSRC and PASB”; 2365, “Stone Matrix Asphalt”; and S-XX (2461) Structural Concrete (Material Delivery management System).

Engineer will use Paper Weight Ticket(s) for 2118.5, “Aggregate Surfacing; 2301.5, “Concrete Pavement”; 2360.5, “Plant Mixed Asphalt Pavement”; 2363.5, “PASSRC and PASB”; 2365.5, “Stone Matrix Asphalt”; and 2461.5, “Structural Concrete” Basis of Payment and for requirements of the Schedule of Materials Control.

Agency MDMS Verification

Engineer will complete Agency MDMS Verification per the schedule outlined in Table SP2061-14.

Table SP2061-14

Schedule for Agency Verification of MDMS Data

| Material Specification Number | Daily Minimum Department QA/Verification (Acceptance) |
| --- | --- |
| 2118, “Aggregate Surfacing” | Third Load, then 1 random review per 1,000 tons |
| 2301, “Concrete Pavement” | Third Load, then 1 random review per 300 cubic yards |
| 2360, “Plant Mixed Asphalt Pavement” | Third Load, then 1 random review per 1,000 tons |
| 2363, “PASSRC and PASB” | Third Load, then 1 random review per 1,000 tons |
| 2365, “Stone Matrix Asphalt” | Third Load, then 1 random review per 1,000 tons |
| 2461, “Structural Concrete” | Third Load, then 1 random review per 100 cubic yards |

METHOD OF MEASUREMENT

The Engineer will measure Material Delivery Management System as a lump sum in accordance with 1901.10, “Lump Sum”.

BASIS OF PAYMENT

Interruptions in availability of data cellular coverage and/or satellite signals used with this system will not result in adjustments to the “Basis of Payment” for any construction items or to Contract time.

The Contract Unit Price for Material Delivery Management System includes data entry of project information; setup of appropriate MDMS components; creation and set up of ICT Platform Workspace in VetaCenter; system setup, mapping and pushing of MDMS Data into Contractor’s MDMS and VetaCenter; internet or satellite connectivity at permanent and portable Sources; setup of Geofences; system monitoring, assigning, distribution and use of truck asset trackers; monitoring of yields rates recorded by MDMS; remote server storage; cloud-based software accessibility and data package plans.

Schedule

Partial payments for Lump Sum Item 2061.601 (Material Delivery Management System) will be made per Table SP2061-15:

Table SP2061-15

Partial Payments Schedule

| When | Percent of Estimated Quantity Completed | Pay Percent of Lump Sum Item |
| --- | --- | --- |
| First Pay Estimate | (Blank) | 10 |
| Subsequent Pay Estimate * | 5 | 15 |
| Subsequent Pay Estimate * | 15 | 30 |
| Subsequent Pay Estimate * | 50 | 50 |
| Subsequent Pay Estimate * | 75 | 75 |
| Final Acceptance of Submittals | 100 | 100 |
| * Percent of Estimated Quantity Completed is based on the pay quantity where the Material Delivery Management System is required: 2360.5, “Plant Mixed Asphalt Pavement”; 2363.5, “PASSRC and PASB”; and 2365.5, “Stone Matrix Asphalt”; 2461.5, “Structural Concrete”. |  |  |

The Department will pay for Material Delivery Management System on the basis of the following schedule:

Item No.	Item	Unit

2061.601	Material Delivery Management System	lump sum